# Advanced Work Zone Traffic Control Training Participant Exam

Name: 	Date:

Final Grade:	Course Location:

Circle the best answer to the following questions (2.5 points each):

- When designing work zones, which of the following should be the designer’s primary concern?

  - The needs of the contractor.

  - The needs of the drivers, other road users and workers.

  - The cost of the project.

  - The duration of the project.

- In which of the following would you find typical traffic control layouts for use in work zone applications in Virginia:

  - Book of Standards.

  - Virginia Work Area Protection Manual.

  - Book of Specifications.

  - ATSSA Quality Standards for Work Zone Traffic Control Devices.

- How many feet will a motorist travel during their perception reaction time (in a work zone) at 45 mph?

A. 88.

B. 110.

C. 276.

D. 331

- Approximately what percentage of all roadway crashes is considered to be due to “distracted driving”?

A. 13%.

B. 34%.

C. 25%.

D. 100%.

- In a work zone, a driver’s perception reaction time (PRT) is estimated to be:

  - 2.5 seconds.

  - 5-7 seconds.

  - 1.47 feet per second.

  - 88 feet per second.

- When designing the use of Type II Drums in a plan, what is the minimum spacing that must be allowed?

  - 3-4 feet.

  - 4-5 feet.

  - 5-6 feet

  - 6-7 feet

- The following are considered parts of a temporary traffic control zone:

  - Advance Warning Area.

  - Transition Area.

  - Buffer Space Area.

  - Work Area.

  - All of the above.

- Of the following elements of the roadway transportation system, which of the following is the most variable and difficult to control?

  - The roadway.

  - The vehicle.

  - The driver.

  - None of the above.

- The minimum length of the longitudinal buffer space depends on:

  - Stopping sight distance as a function of speed.

  - The length of the workspace

  - The time of day.

  - The duration of the project.

- The length of the Longitudinal Buffer in a lane closure on a multilane secondary roadway posted 50 mph is 	:

A. 225 – 260 feet. B. 310 – 320 feet. C. 425 - 445 feet. D.  450 – 475 feet.

- The work duration of a Short-Term Stationary work zone is:

  - An activity that moves intermittently (0-15 minutes) or continuously.

  - An activity that occupies a location from 15 minutes up to 1 hour.

  - An activity that occupies a location more than 3 consecutive days.

  - A daytime activity that occupies a location for more than 1 hour, but less than 12 hours.

- Which of the following require special consideration by the TCP designer?

  - Senior citizen retirement home.

  - School zones and playgrounds.

  - Steep down grade.

  - All of the above require special consideration.

- When designing and installing a work zone, motorists speeds affect the following:

  - Distance between warning signs, length of merging taper, spacing of cones.

  - How many flashing lights required, number of arrow panels needed, height of cones?

  - The location and duration of construction.

  - Number of signs you need, and where you place them.

- The “Clear Zone” for an ordinary road with speed posted at 45mph is:

  - 4 feet.

  - 15 feet.

  - 25 feet.

  - 27 feet.

- The suggested sign spacing for signs on an urban 45 m.p.h. roadway is:

  - 100 – 200 feet.

  - 500 – 800 feet.

C.  350 – 500 feet.

D.  1300 – 1500 feet.

- Which of the following are used in unmanned work zones?

  - Cones – Group 2 Channelizing Device.

  - Post-mounted signs.

  - Drums – Group 2 Channelizing Device.

  - Traffic Control Plans.

- On roads that are not limited access, the formula for determining shifting tapers is :

  - L x .75

  - L minimum.

C.  1/3 L.

- 100 feet per lane.

- A "Merging" Taper is used to:

  - Change alignment of an existing lane.

  - Move traffic out of a normal travel lane.

  - Close off a shoulder.

  - Allow traffic to return to normal travel lanes.

- For a 50-mph roadway, which of the following tapers is the shortest?

  - Lane merging taper.

  - Flagging two-way traffic taper.

  - Lane shifting taper.

  - Shoulder taper.

- The posted speed is 60 mph, the minimum length of a merging taper on a limited access highway is:

- 330 feet.

- 660 feet.

- 800 feet.

11. 1000 feet.

- What is the minimum length of a merging taper required to close a 12-foot lane on a 50-mph multilane roadway?

  - 600 feet.

  - 660 feet.

  - 720 feet.

  - 50-100 feet maximum.

- A 1,000-ft long tangent is part of a traffic control plan. If the spacing between the cones is 40 feet, how many cones (minimum) would be needed?

A. 10.

B. 15.

C. 26.

D. 30.

- The two standard sizes of arrow panels which should be used on Virginia roadways are:

  - Type A and B.

  - Type B and C.

  - Type A and C.

  - Type B and D.

- Pavement markers are installed on 	centers in all tangent areas. A.	80’.

B.	40’.

C.	20’.

D.	10’.

- The formula used to determine when barrier service is required to protect a fixed object considers:

  - Cost of barrier, pavement marking, and marking eradication.

  - Average Daily Traffic (ADT), exposure time, and length of the hazard.

  - Weight of barrier, effect on traffic, and barrier deflection.

  - Number of laborers, delivery trucks, and delineator panels.

- The barrier transition slope ratio for concrete barriers placed in a right lane closure operation on a limited access highway with 50,000 vehicles per day and posted 65 mph is:

A. 28:1.

B. 30:1.

C. 20:1.

D. 17:1.

- None of the above.

- Which projects should have an approved Traffic Control Plan?

  - Only projects on the State’s highway system.

  - Only long-term projects.

  - Only projects on high-speed roadways.

  - Only on Category C projects.

- When arrow boards are used to close multiple lanes only 1 may be used in:

  - Shifting tapers.

  - Nighttime lane closure.

  - High-speed freeways lane closure.

  - Each closed lane.

- For a double lane closure on a 6-lane, 65 mph, 12-ft lane freeway, two merging tapers are required. What is the minimum separation between the two tapers?

  - 1000 feet.

  - 2000 feet.

  - 1580 feet.

  - 780 feet.

- Which of the following statements is not a “constructability” issue?

  - The access requirements of the workspace.

  - Drainage.

  - Room to maneuver equipment.

  - The construction diary.

Circle T for True or F for False at the end of each statement (2.5 points each):

- In the MUTCD (and the Virginia Work Area Protection Manual), the term "may" is considered a mandatory condition.	T  or F

- The Buffer Space Area is set aside for the placement of equipment, materials, and vehicles to keep the work area free.	T  or F

- The traffic space is the portion of the highway in which road users are routed through the work area.	T  or F

- The work duration of a “moble” work zone last more than 30 minutes to 1 hour.

T  or F

- The five basic requirements that traffic control devices should meet to be effective are: fulfill a need, command attention, convey a clear - simple meaning, command respect, and give adequate time for response.	T  or F

- The Shoulder Closure with Barrier and Lane Shift diagram is found on page 6F-10 of the 2011 Revision 2 Virginia Work Area Protection Manual.	T  or F

- The proper placement of arrow panels in lane closures is halfway up the taper in the middle of the closed lane.	T  or F

- Black traffic paint is an acceptable form of pavement marking eradication.

T  or F

- A Truck Mounted Attenuator is required when closing a lane on a multi-lane roadway posted 35 mph, and on all limited access highway lane closures.

T  or F

- TTC-7.1, pages 6H-25 and 6H-26 is an inside lane closure on a four-lane roadway.

T  or F